Homework 3

1. Write a program to clear Register B, and then add the number three to B seven times. Use the BNE instruction with DECA.

2. Write a program to clear Register A, add five to Register A 10 times, and place the result in RAM Location $2000. Use a RAM location for holding the counter.

3. What is the size of each instruction?
I. LDAA     #50
II. BEQ      there
III. STAA    $2000
IV. DECB
V. LDAB     0, X

4. Write the sequence of instructions to save Registers A, Y, and X in the Stack with the given order. Write the sequence of instructions to retrieve those values back.
Saving A, Y, and X in Stack:
 
1 __________
 
2 __________
 
3 __________
 
 
Retrieving values back:
 
1 __________
 
2 __________
 
3 __________

5. Show code for a nested loop to perform an action 1,000 times.